This FAQ archived and provided free as a courtesy by

The Providence Cooperative

http://www.providenceco-op.com

=======================================================================

Version 2.2

Updated 7/19/98

Supersedes: Version 2.0

Water Treatment FAQ

Version 2.2

By

Patton Turner

Acknowledgements:  Thanks to the following people for making additions,

corrections, or suggestions:  Richard DeCastro,  decastro@netcom.com;

Henry Schaffer, hes@unity.ncsu.edu; Alan T. Hagan,

athagan@sprintmail.com;  Logan Van Leigh, loganv@earthlink.net;  Carl

Stiles, blitz1@airmail.net .

Alan also provided the wording for the disclaimer and copyright notice.

Copyright 1997, 1998.  All rights reserved.

Excluding contributions attributed to specific individuals all material

in this work is copyrighted to Patton Turner and all rights are

reserved.  This work may be copied and distributed freely as long as the

entire text, my and the contributor's names and this copyright notice

remains intact, unless my prior permission has been obtained.  This FAQ

may not be distributed for financial gain, included in commercial

collections or compilations, or included as a part of the content of any

web site without prior, express permission from the author.

=======================================================================

DISCLAIMER:  Safe and effective water treatment requires attention to

detail and proper equipment and ingredients.  The author makes no

warranties and assumes no responsibility for errors or omissions in the

text, or damages resulting from the use or misuse of information

contained herein

Placement of or access to this work on this or any other site does not

mean the author espouses or adopts any political, philosophical or

meta-physical concepts that may also be expressed wherever this work

appears.

=======================================================================

Water Storage

-Quantity-

A water ration of as little as a pint per day has allowed life raft

survivors to live for weeks, but a more realistic figure is 1 gallon per

person per day for survival.  4 gallons per person/day will allow

personal hygiene, washing of dishes, counter tops, etc.  5 to 12 gallons

per day would be needed for a conventional toilet, or 1/2 to two gallons

for a pour flush latrine.  For short-term emergencies, it will probably

be more practical to store paper plates and utensils, and minimize food

preparation, than to attempt to store more water.

In addition to stored water, there is quite a bit of water trapped in

the piping of the average home.  If the municipal water system was not

contaminated before you shut the water off to your house, this water is

still fit for consumption without treatment.  To collect this water,

open the lowest faucet in the system, and allow air into the system from

a second faucet.  Depending on the diameter of the piping, you may want

to open every other faucet, to make sure all of the water is drained.

This procedure will usually only drain the cold water side, the

hot-water side will have to be drained from the water heater.  Again,

open all of the faucets to let air into the system, and be prepared to

collect any water that comes out when the first faucet is opened.

Toilet tanks (not the bowls) represent another source of water if a

toilet bowl cleaner is not used in the tank.

Some people have plumbed old water heaters or other tanks in line with

their cold water supply to add an always rotated source of water.  Two

cautions are in order:  1) make sure the tanks can handle the pressure

(50 psi min.), and 2) if the tanks are in series with the house

plumbing, this method is susceptible to contamination of the municipal

water system.  The system can be fed off the water lines with a shutoff

valve (and a second drain line), preventing the water from being

contaminated as long as the valve was closed at the time of

contamination.

Water can only be realistically stored for short-term emergencies, after

that some emergency supply of water needs to be developed.

Water Collection

-Wells-

Water can only be moved by suction for an equivalent head of about 20'.

After this cavitation occurs, that is the water boils off in tiny

bubbles in the vacuum created by the pump rather than being lifted by

the pump.  At best no water is pumped, at worst the pump is destroyed.

Well pumps in wells deeper than this work on one of the following

principles:

1) The pump can be submerged in the well, this is usually the case for

deep well pumps. Submersible pumps are available for depths up 1000

feet.

2) The pump can be located at the surface of the well, and two pipes go

down the well: one carrying water down, and one returning it.  A jet

fixture called an ejector on the bottom of the two hoses causes well

water to be lifted up the well with the returning pumped water.  These

pumps must have an efficient foot valve as there is no way for them to

self-prime.  These are commonly used in shallow wells, but can go as

deep as 350 feet.  Some pumps use the annular space between one pipe and

the well casing as the second pipe this requires a packer (seal) at the

ejector and at the top of the casing.

3) The pump cylinder can be located in the well, and the power source

located above the well.  This is the method used by windmills and most

hand pumps.  A few hand pumps pump the water from very shallow wells

using an aboveground pump and suction line.  A variety of primitive, but

ingenious, pump designs also exist.  One uses a chain with buckets to

lift the water up.  Another design uses a continuous loop rope dropping

in the well and returning up a small diameter pipe.  Sealing washers are

located along the rope, such that water is pulled up the pipe with the

rope.  An ancient Chinese design used knots, but modern designs designed

for village level maintenance in Africa use rubber washers made from

tires, and will work to a much greater depth.

Obviously a bucket can be lowered down the well if the well is big

enough, but this won't work with a modern drilled well.  A better idea

for a drilled well is to use a 2' length or so of galvanized pipe with

end caps of a diameter that will fit in the well casing.  The upper cap

is drilled for a screw eye, and a small hole for ventilation.  The lower

end is drilled with a hole about half the diameter of the pipe, and on

the inside a piece of rigid plastic or rubber is used as a flapper

valve. This will allow water to enter the pipe, but not exit it.  The

whole assembly is lowered in the well casing, the weight of the pipe

will cause it to fill with water, and it can then be lifted to the

surface. The top pipe cap is there mostly to prevent the pipe from

catching as it is lifted.

-Springs-

Springs or artesian wells are ideal sources of water.  Like a

conventional well, the water should be tested for pathogens, VOCs

(Volatile Organic Compounds such as fuel oil or benzene), pesticides and

any other contaminants found in your area.  If the source is a spring it

is very important to seal it in a spring box to prevent the water from

becoming contaminated as it reaches the surface.  It is also important

to divert surface runoff around the spring box.  As with a well, you

will want to periodically treat the spring box with chlorine,

particularly if the spring is slow moving.  The spring may also be used

for keeping food cool if a spring-house is built.  If this is the case,

it is still recommended to build a spring box inside the house to obtain

potable water.

-Surface water-

Most US residents served by municipal water systems supplied with

surface water, and many residents of underdeveloped countries rely on

surface water.  While surface water will almost always need to be

treated, a lot of the risk can be reduced by properly collecting the

water.  Ideal sources of water are fast flowing creeks and rivers which

don't have large sources of pollution in their watershed.  With the

small amounts of water needed by a family or small group, the most

practical way to collect the water is though an infiltration gallery or

well.  Either method reduces the turbidity of the collected water making

it easy for later treatment.

Water Purification

-Contaminants-

Heavy Metals

Heavy metals are only a problem is certain areas of the country.  The

best way to identify their presence is by a lab test of the water or by

speaking with your county health department. Unless you are down stream

of mining trailings or a factory, the problem will probably affect the

whole county or region.  Heavy metals are unlikely to be present in

sufficient levels to cause problems with short-term use.

Turbidity

Turbidity refers to suspended solids, i.e. muddy water, is very turbid.

Turbidity is undesirable for 3 reasons:  1) aesthetic considerations 2)

solids may contain heavy metals, pathogens or other contaminants, 3)

turbidity decreases the effectiveness of water treatment techniques by

shielding pathogens from chemical or thermal damage, or in the case of

UV treatment, absorbing the UV light itself.

Organic compounds

Water can be contaminated by a number of organic compound such as

chloroform, gasoline, pesticides, and herbicides.  These contaminants

must be identified in a lab test.  It is unlikely ground water will

suddenly become contaminated unless a quantity of chemicals is allowed

to enter a well or penetrating the aquifer.  One exception is when the

aquifer is located in limestone.  Not only will water flow faster

through limestone, but the rock is prone to forming vertical channels or

sinkholes that will rapidly allow contamination from surface water.

Surface water may show great swings in chemical levels due to

differences in rainfall, seasonal crop cultivation, and industrial

effluent levels

-Pathogens-

Protozoa

Protozoa cysts are the largest pathogens in drinking water, and are

responsible for many of the waterborne disease cases in the US.

Protozoa cysts range is size from 2 to 15 microns (a micron is one

millionth of a meter), but can squeeze through smaller openings.  In

order to insure cyst filtration, filters with a absolute pore size of

1 micron or less should be used.  The two most common protozoa pathogens

are Giardia lamblia (Giardia) and Cryptosporidium (Crypto).  Both

organisms have caused numerous deaths in recent years in the US, the

deaths occurring in the young and elderly, and the sick and immune

compromised. Many deaths were a result of more than one of these

conditions.  Neither disease is likely to be fatal to a healthy adult,

even if untreated. For example in Milwaukee in April of 1993, of 400,000

who were diagnosed with Crypto, only 54 deaths were linked to the

outbreak, 84% of whom were AIDS patients.  Outside of the US and other

developed countries, protozoa are responsible for many cases of amoebic

dysentery, but so far this has not been a problem in the US, due to

better wastewater treatment.  This could change during a survival

situation.  Tests have found Giardia and/or Crypto in up to 5% of

vertical wells and 26% of springs in the US.

Bacteria

Bacteria are smaller than protozoa and are responsible for many diseases

such as typhoid fever, cholera, diarrhea, and dysentery.  Pathogenic

bacteria range in size from 0.2 to 0.6 microns, and a 0.2 micron filter

is necessary to prevent transmission.  Contamination of water supplies

by bacteria is blamed for the cholera epidemics which devastate

undeveloped countries from time to time.  Even in the US, E. coli is

frequently found to contaminate water supplies.  Fortunately E. coli is

relatively harmless as pathogens go, and the problem isn't so much with

E. coli found, but the fear that other bacteria may have contaminated

the water as well.  Never the less, dehydration from diarrhea caused by

E. coli has resulted in fatalities.

Viruses

Viruses are the 2nd most problematic pathogen, behind protozoa.  As with

protozoa, most waterborne viral diseases don't present a lethal hazard

to a healthy adult.  Waterborne pathogenic viruses range in size from

0.020-0.030 microns, and are too small to be filtered out by a

mechanical filter.  All waterborne enteric viruses affecting humans

occur solely in humans, thus animal waste doesn't present much of a

viral threat.  At the present viruses don't present a major hazard to

people drinking surface water in the US, but this could change in a

survival situation as the level of human sanitation is reduced.  Viruses

do tend to show up even in remote areas, so case can be made for

eliminating them now.

Physical Treatment

Heat Treatment

Boiling is one guaranteed way to purify water of all pathogens.  Most

experts feel that if the water reaches a rolling boil it is safe.  A few

still hold out for maintaining the boiling for some length of time,

commonly 5 or 10 minutes, plus an extra minute for every 1000 feet of

elevation.  If one wishes to do this, a pressure cooker would allow the

water to be kept at boiling with out loosing the heat to evaporation.

One reason for the long period of boiling may be to inactivate bacterial

spores (which can survive boiling), but these spore are unlikely to be

waterborne pathogens.

African aid agencies figure it takes 1 kg of wood to boil 1 liter of

water.  Hardwoods and efficient stoves would improve on this.

Water can also be treated at below boiling temperatures, if contact time

is increased.  A commercial unit has been developed that treats 500 gals

of water per day at an estimated cost of $1/1000 gallons for the energy.

The process is similar to milk pasteurization, and holds the water at

161 deg F for 15 seconds.  Heat exchangers recover most of the energy

used to warm the water.  Solar pasteurizers have also been built that

would heat three gallons of water to 65deg C and hold the temperature

for an hour.  A higher temperature could be reached if the device was

rotated east to west during the day to follow the sunlight.

Regardless of the method, heat treatment does not leave any form of

residual to keep the water free of pathogens in storage.

Reverse Osmosis.

Reverse osmosis forces water, under pressure, through a membrane that is

impermeable to most contaminants.  The most common use is aboard boats

to produce fresh water from salt water. The membrane is somewhat better

at rejecting salts than it is at rejecting non-ionized weak acids and

bases and smaller organic molecules (molecular weight below 200).  In

the latter category are undissociated weak organic acids, amines,

phenols, chlorinated hydrocarbons, some pesticides and low molecular

weight alcohols.  Larger organic molecules, and all pathogens are

rejected.  Of course it is possible to have a imperfection in the

membrane that could allow molecules or whole pathogens to pass through.

Using reverse osmosis to desalinate seawater requires considerable

pressure (1000 psi) to operate, and for a long time only electric models

were available.  Competing for a contract to build a hand powered model

for the Navy, Recovery Engineering designed a model that could operate

by hand, using the waste water (90 percent of the water is waste water,

only 10% passes through the filter) to pressurize the back side of the

piston.  The design was later acquired by PUR.  While there is little

question that the devices work well, the considerable effort required to

operate one has been questioned by some survival experts such as Michael

Greenwald, himself a survivor of a shipwreck. On the other hand the

people who have actually used them on a life raft credit the

availability of water from their PUR watermaker for their survival.

PUR manual watermakers are available in two models:

The Survivor 06 ($500) produces 2 pints per hour, and

The Survivor 35 ($1350) produces 1.4 gal/hr.  The latter model is also

available as the Power Survivor 35 ($1700), which produces the same

water volume from 4 Amps of 12 VDC, and can be disconnected and used as

a hand held unit.

A number of manufactures, including PUR, make DC powered models for

shipboard use. PUR recommends replacing the O rings every 600 hours on

its handheld units, and a kit is available to do this.  Estimates for

membrane life vary, but units designed for production use may last a

year or more. Every precaution should be taken to prevent petroleum

products from contacting the membrane as they will damage or destroy the

membrane. The prefilter must also be regularly changed, and the membrane

may need to be treated with a biocide occasionally

Reverse osmosis filter are also available that will use normal municipal

or private water pressure to remove contaminates from water, as long as

they aren't present in the levels found in sea water.

The water produced by reverse osmosis, like distilled water, will be

close to pure H2O. Therefore mineral intake may need to be increased to

compensate for the normal mineral content of water in much of the world.

Distillation

Distillation is the evaporation and condensation of water to purify

water.  Distillation has two disadvantages:

1) A large energy input is required and

2) If simple distillation is used, chemical contaminants with boiling

points below water will be condensed along with the water.

Distillation is most commonly used to remove dissolved minerals and

salts from water.

The simplest form of a distillation is a solar still.  A solar still

uses solar radiation to evaporate water below the boiling point, and the

cooler ambient air to condense the vapor.  The water can be extracted

from the soil, vegetation piled in the still, or contaminated water

(such as radiator fluid or salt water) can be added to the still.  While

per still output is low, they are an important technique if water is in

short supply

Other forms of distillation require a concentrated heat source to boil

water which is then condensed.  Simple stills use a coiling coil to

return this heat to the environment.  These can be improvised with a

boiler and tight fitting lid and some copper tubing (Avoid using lead

soldered tubing if possible).  FEMA suggests that, in an emergency, a

hand towel can be used to collect steam above a container of boiling

water.  More efficient distillations plants use a vapor compression

cycle where the water is boiled off at atmospheric pressure, the steam

is compressed, and the condenser condenses the steam above the boiling

point of the water in the boiler, returning the heat of fusion to the

boiling water.  The hot condensed water is run through a second heat

exchanger which heats up the water feeding into the boiler.  These

plants normally use an internal combustion engine to run the compressor.

Waste heat from the engine, including the exhaust, is used to start the

process and make up any heat loss.  This is the method used in most

commercial and military desalinization plants

Inflatable solar stills are available from marine supply stores, but

avoid the WW2 surplus models, as those who have used them have had a

extremely high failure rate.  Even new inflatable solar stills may only

produce from 30-16 oz under actual conditions, compared to a rating of

48 oz/day under optimum conditions.

Jade Mountain also offers the following portable models in travel cases:

Traveler (WC106)	1 gpd, 23 lb., 24x26x10 folded	$  695

Base Camp (WC107)   2 gpd, 51 lb., 48x48x4 folded      $  895

Safari (WC108)                48x48x5        $1095

A ruggedized version of the Base Camp above

Microfilters

Microfilters are small-scale filters designed to remove cysts, suspended

solids, protozoa, and in some cases bacteria from water.  Most filters

use a ceramic or fiber element that can be cleaned to restore

performance as the units are used.  Most units and almost all made for

camping use a hand pump to force the water through the filter.  Others

use gravity, either by placing the water to be filtered above the filter

(e.g. the Katadyn drip filter), or by placing the filter in the water,

and running a siphon hose to a collection vessel located below the

filter (e.g. Katadyn siphon filter). Microfilters are the only method,

other than boiling, to remove Cryptosporidia.  Microfilters do not

remove viruses, which many experts do not consider to be a problem in

North America.  Despite this the Katadyn microfilter has seen

considerable use around the world by NATO-member militaries, WHO, UNHCR,

and other aid organizations.  Microfilters share a problem with charcoal

filter in having bacteria grow on the filter medium.  Some handle this

by impregnating the filter element with silver such as the Katadyn,

others advise against storage of a filter element after it has been

used.  The Sweetwater Guardian suggests using a freezer for short-term

storage

Many microfilters may include silt prefilters, activated charcoal

stages, or an iodine resin. Most filters come with a stainless steel

prefilter, but other purchased or improvised filters can be added to

reduce the loading on the main filter element.  Allowing time for solids

to settle, and/or prefiltering with a coffee filter will also extend

filter life.  Iodine matrix filters will kill viruses that will pass

through the filter, and if a charcoal stage is used it will remove much

of the iodine from the water.  Charcoal filters will also remove other

dissolved natural or manmade contaminates. Both the iodine and the

charcoal stages do not indicate when they reach their useful life, which

is much shorter than the filter element.  If you are depending on the

stage for filtering the water you will have to keep up with how much

water passes through it.

New designs seem to be coming out every month.  The best selling brands

seem to be the PUR, and Sweetwater Guardian.  The Katadyn doesn't sell

as well to outdoor enthusiasts due to its high cost, but for years it

was state of the art for water purification and still has a loyal

following, especially among professionals in relief work.  Below is the

data on a few of the more common units, for a excellent field test of

some common units, see the December 96 issue of Backpacker magazine.

Note that the first price is for the filter, the second for the

replacement filter.  The weight is from manufacturer's literature if it

was not listed in the Backpacker article.  Filter life is from

manufacturer’s literature and should be taken with a grain of salt.

Basic Designs Ceramic Filter Pump ($29/$15, 8 oz.)  Cheap flimsy filter,

claimed to filter up to 500 gallons with a 0.9 micron ceramic filter.

Not EPA rated, may not have passed independent lab tests, prone to

damage, filter element must be submerged in water.

General Ecology- First Need Deluxe ($70/$30, 20 oz) This filter uses a

structured matrix micro strainer, though General Ecology won't reveal

what the structure is.  It has survived independent lab tests, and

filters particles to .4 microns, while actually removing viruses (the

only filter capable of doing this) through electrostatic attraction.

The filter cartridges can't be cleaned (other than by back flushing),

but are good for 100 gallons.  Pump design isn't the best.  Other models

are available from the manufacturer.

Katadyn PF  ($295/$145, 22.7 oz).  The original microfilter using a 0.2

micron silver impregnated ceramic candle.  An extremely thick filter

allows it to be cleaned many times for up to 14,000 gallons capacity.

While the Katadyn seems well made, one reader of this list reported

breaking the candle, and Backpacker Magazine broke the case during a

field test. The pump, while probably indestructible, is somewhat slow

and hard to use, requiring 20 lbs. of force on a small handle.  The PF

also lacks a output hose as the Katadyn engineers felt if would be a

source of contamination.

Katadyn Combi ($185/$75 (ceramic)/$19 (carbon), 29 oz) A cheaper version

of the PF incorporating both ceramic and carbon stages.  Much faster

filter than the PF.

Katadyn Minifilter ($139/$59, 8.3 oz) A smaller and cheaper version of

the PF, easier to pump, but generally not well received. Good for 200

gallons.

Katadyn Expedition ($680/$77, 13 lb.)  Similar filter to the PF (exact

same cartridge as the Drip Filter Below), but designed for much higher

production, stainless steel case with spade type D handle, produces 0.75

gpm.  Filter good for 26,000 gallons.

Katadyn Drip Style Filter ($240, $77, 12.5 lb.)  Filter elements similar

to those in the PF are mounted vertically in top 3 gallon plastic

bucket, water drips through filters into second 3 gallon bucket with

faucet. 1 qt, per hour with the 2 filters included, a third filter can

be added to increase rate 50%.  Each filter good for 13,000 gallons.

The mounting hardware for the filters is available for $10 to allow you

to make your own filter of what ever size is needed.  Each mounting kit

requires a ½” hole in the bottom of the raw water container.

Katadyn Siphon Filter  ($92, 2 lb.) Similar design to PF filter element,

but a siphon hose replaces the pump, filters 1-2 quarts per hour (allow

1 hour for the filter to "prime" itself via capillary action), but

multiple filters can be used in the same container.  Collection vessel

must be lower than raw water container.  Good for 13,000 gallons.

MSR Miniworks ($59/$30, 14 oz) MSR's smaller filter, using a 0.3 micron

ceramic element.  Pump is well designed, and easy to use.  Main drawback

is that the clean water discharge is from the bottom of the filter, and

no hose is provided.  While the bottom is threaded for a Nalgene bottle,

it is a pain in the butt to fill a canteen or 2 liter bottle.  Claimed

to filter 100 gallons, Backpacker Magazine feels this may be one of the

few filters without a grossly inflated rating

MSR Waterworks ($140/$30/$30, 17 oz) MSR's first filter with a 0.2

micron ceramic and membrane stage and a carbon stage.  Other wise

similar to the Miniworks.

PUR Pioneer ($30/$4, 8 oz), newly introduced low-end microfilter.  0.5

micron, 1 lpm filter rate, 12 gallon capacity

PUR Hiker ($50/$20, 12 oz)  PUR's microfilter only design, filters to .5

micron.  Well liked, as are the other PUR filters. Very compact. 200

gallon capacity

PUR Scout ($70/$35/$15, 12 oz) Combines a iodine resin stage, a 1.0

micron filter, and a activated charcoal filter. 200 gallon capacity

PUR Explorer ($130/$45, 22 oz)  PUR's top of the line model. Bulky, but

well made, with a high output (1.4  lpm, faster than any of the hand

held models listed and one of the easiest to pump)  Has a 1.0 micron

filter plus a iodine resin stage, 300 gallon capacity

Sweetwater Walkabout($35/$13, 8.5 oz.) Sweetwater's low end filter, 0.2

micron, .7 lpm, 100 gal capacity

Sweetwater Guardian ($60/$20, 11 oz) Uses a glass fiber and carbon

filter, filters to .2 micron, claimed to last for 200 gallons.  An

iodine resin stage can be added that will kill viruses, and will last

for 90 gallons.  Pump is well designed, but it takes a few seconds to

pull a captive pin to fold for storage. Available in white or OD.

Timberline Eagle ($20/$13, 8 oz) At 1 micron, this filter only does

protozoa, but is much easier to pump, lighter, and cheaper.  Filter is

attached to pump, and must rest (but doesn't have to be submerged) in

water to be purified.  Looks flimsy, but seems to hold up.  Claimed to

last for 100 gallons.

It is also possible to build your own microfilter using diatomaceous

earth, sold for swimming pool filters (DE).  Usually pressure is

required to achieve a reasonable flow rate.  A DE filter will remove

turbidity as well as pathogens larger than 1 micron.

Slow Sand Filter

Slow sand filters pass water slowly through a bed of sand.  Pathogens

and turbidity are removed by natural die-off, biological action, and

filtering.  Typically the filter will consist of 24 inches of sand, then

a gravel layer in which the drain pipe is embedded.  The gravel doesn't

touch the walls of the filter so that water  can't run quickly down the

wall of the filter and into the gravel. Building the walls with a rough

surface also helps.  A typical loading rate for the filter is 0.2

meters/hour day (the same as .2 m^3/m^2 of surface area).  The filter

can be cleaned several times before the sand has to be replaced.

Slow sand filter construction information:  Slow sand filters should

only be used for continuous water treatment.  If a continuous supply of

raw water can't be insured (say using a holding tank), then another

method should be chosen.  It is also important for the water to have as

low turbidity (suspended solids) as possible.  Turbidity can be reduced

by changing the method of collection (for example, building an

infiltration gallery, rather than taking water directly from a creek),

allowing time for the material to settle out (using a raw water tank),

prefiltering or flocculation (adding a chemical such as alum to cause

the suspended material to floc together.)

The SSF filter itself is a large box, at least 1.5 meters high.  The

walls should be as rough as possible to reduce the tendency for water to

run down the walls of the filter, bypassing the sand. The bottom layer

of the filter is a gravel bed in which a slotted pipe is placed to drain

off the filtered water.  The slots or the gravel should be no closer

than 20 cm to the walls. again to prevent the water from bypassing the

sand.

The sand for a SSF needs to be clean and uniform, and of the correct

size.  The sand can be cleaned in clean running water , even if it is in

a creek.  The ideal specs on sand are effective size (sieve size through

which 10% of the sand passes) between 0.15 and 0.35 mm, uniformity

coefficient (ratio of sieve sizes through which 60% pass and through

which 10% pass) of less than 3, Maximum size of 3 mm, and minimum size

of 0.1 mm.

The sand is added to a SSF to a minimum depth of 0.6 meters.  Additional

thickness will allow more cleanings before the sand must be replaced.

0.3 to 0.5 meters of extra sand will allow the filter to work for 3-4

years.  An improved design uses a geotextile layer on top of the sand to

reduce the frequency of cleaning.  The outlet of a SSF must be above the

sand level, and below the water level.  The water must be maintained at

a constant level to insure an even flow rate throughout the filter.  The

flow rate can be increased by lowering the outlet pipe, or increasing

the water level.  One common idea for maintaining the water level is to

use a elevated raw water tank or pump, and a ball valve from a toilet.

While the SSF will begin to work at once, optimum treatment for

pathogens will take a week or more.  During this time the water should

be chlorinated if at all possible (iodine can be substituted).  After

the filter has stabilized, the water should be safe to drink, but

chlorinating of the output is still a good idea, particularly to prevent

recontamination.

As the flow rate slows down the filter will have to be cleaned by

draining and removing the top few inches of sand.  If a geotextile

filter is used, only the top ½” may have to be removed.  As the filter

is refilled, it will take a few days for the biological processes to

reestablish themselves.

Activated Charcoal Filter

Activated charcoal filters water through adsorption, chemicals and some

heavy metals are attracted to the surface of the charcoal, and are

attached to it.  Charcoal filters will filter some pathogens though they

will quickly use up the filter adsorptive ability, and can even

contribute to contamination as the charcoal provides an excellent

breeding ground for bacteria and algae.  Some charcoal filters are

available impregnated with silver to prevent this, though current

research concludes that the bacteria growing on the filter are harmless,

even if the water wasn't disinfected before contacting the filter.  The

only filter I know of that uses only activated charcoal, and doesn't

required pressurized water is the Water Washer ($59).  Available from

the Survival Center.

Activated charcoal can be used in conjunction with chemical treatment.

The chemical (iodine or chlorine) will kill the pathogens, while the

carbon filter will remove the treatment chemicals.  In this case, as the

filter reaches its capacity, a distinctive chlorine or iodine taste will

be noted.

Activated charcoal can be made at home, though the product will be of

varying quality compared to commercial products.  Either purchased or

homemade charcoal can be recycled by burning off the molecules adsorbed

by the carbon (The won't work with heavy metals of course.)

The more activated charcoal in a filter, the longer it will last.  The

bed of carbon must be deep enough for adequate contact with the water.

Production designs use granulated activated charcoal (effective size or

0.6 to 0.9 mm for maximum flow rate.  Home or field models can also use

a compressed carbon block or powered activated charcoal (effective size

0.01) to increase contact area.  Powered charcoal can also be mixed with

water and filtered out later.  As far as life of the filter is

concerned, carbon block filters will last the longest for a given size,

simply due to their greater mass of carbon.  A source of pressure is

usually needed with carbon block filters to achieve a reasonable flow

rate.

Sol-Air Water Treatment

If sufficient dissolved oxygen is available, sunlight will cause the

temporary formation of reactive forms of oxygen such as hydrogen

peroxide and oxygen free radicals.  This form of water treatment is

called solar photooxidative disinfection or sol-air water treatment.

Sol-Air water treatment has been shown to dramatically reduce the level

of fecal coliform bacteria.  There is some evidence that other bacteria

and viruses may be affected also.  While not as reliable as other

methods, it does offer a low-tech solution in emergencies.  Sol-Air

treatment requires bright sunlight, and has been shown to be effective

when ever the sun causes a distinct shadow to be cast. Exposure to 4.5

hours of bright sunlight has been shown to cause a thousand fold

reduction in fecal coliforms in lab tests

In order for Sol-Air to be effective, oxygen must be present.

Experiments have shown that shaking a bottle filled 3/4 with air will

restore oxygen levels to near saturation.  As the treatment continues,

some of the oxygen will come out of solution, while other oxygen will be

consumed by the killed pathogens, so the shaking should be repeated

every few hours.  Data shows that maximum activity occurs when the water

temperature is above 50deg C (122deg F), so this method may be

unsuitable in colder climates unless special solar collectors are used.

Either glass or plastic bottles may be used.  Plastic bottles will allow

short wave ultraviolet radiation to pass, increasing the rate of

microbial inactivation, but may yellow with age, reducing light

transmission, and may leach plasticizers into the water at the elevated

temperatures that will occur.  The leaching of plasticizers can be

reduced by using bottles of PET  (polyethlyene terephtalate) rather than

PVC.  Glass bottles on the other hand are more durable.  Research has

used bottles with 2 liters of capacity, but if the water is free of

turbidity, larger containers can be used. Plastic bags, or some sort of

flat glass container represent the ideal container as this maximizes the

solar energy received per ounce of water.

Bottles should be filed 3/4 full in the early morning with water as free

of turbidity as possible.  After capping the bottles should be shaken

vigorously for a few minutes then placed upright in the sun, where they

will be not be shaded later in the day.  The shaking should be repeated

at least three times during the day.  At the end of the day the water

should be reasonably freed of bacteria, though it is most practical to

let the water cool for consumption the following day. Each day a new

batch should be treated due to the lack of a residual disinfected.

After consumption of the water the bottle should be air dried to prevent

algae growth with continual use.

Improvised Mechanical Filter

If the materials aren’t available to build a slow sand filter, or some

other means of water treatment is preferred, it may still be

advantageous to mechanically filter the water before treating it with

chemicals or passing through a microfilter.  Generally the idea is to

allow the water to flow as slowly as possible through a bed of sand.  In

a municipal water treatment plant this is called a rapid sand filter.

The particular design below is included, because the designer, a

research engineer at Oak Ridge National Laboratories, found it

particularly effective at removing fallout from water. The filter will

do little or nothing to remove pathogens, though removing suspended

solids allow others water treatment methods to work more effectively.

Expedient water filter, from Nuclear War Survival Skills, Cresson

Kearny, ORNL

1) Perforate the bottom of a 5 gallon bucket, or similar container with

a dozen nail holes even spread over a 4" diameter circle in the center

of the container.

2) Place a 1.5" layer of small stones or pebbles in the bottom of the

can.  If pebbles aren’t available, marbles, clean bottle caps, twisted

coat hangers or clean twigs can be used.

3) Cover the pebbles with one thickness of terrycloth towel, burlap

sackcloth, or other porous cloth. Curl the cloth in a roughly circular

shape about three inches larger then the diameter of the can.

4) Take soil containing some clay (pure clay isn't porous enough, pure

sand is too porous) from at least 4” below the surface of the ground

(nearly all fallout particles remain near the surface except after

disposition on sand or gravel.)

5) Pulverize the soil, then gently press it in layers over the cloth

that covers the pebbles, so that the cloth is held snugly against the

walls of the can.   The soil should be 6-7" thick.

6) Completely cover the surface of the soil layer with one thickness of

fabric as porous as a bath towel.  This is to keep the soil from being

eroded as water is being poured into the filter.  A dozen small stones

placed on the cloth near it's edges will secure it adequately.

7) Support the filter on rocks or sticks placed across the top of a

container that is larger then the filter can (such as a dishpan)

The contaminated water should be poured into the filter can, preferably

after allowing it to settle as described below.  The filtered water

should be disinfected by some method.

If the 6 or 7 inches of filtering soil is a sandy clay loam, the filter

will initially deliver about 6 quarts/hour.  If the filter is any faster

than this then the fabric layer needs to be removed and the soil

compressed more.  The filtering rate will drop over time as the filter

begins to clog up.  When this happens the top 1/2" of soil can be

removed to increase the filtering rate.  After 50 or so quarts, the

filter will need to be rebuilt with fresh soil.

As with any filter, optimum performance will be achieved if sediment in

the water will be allowed to settle out before passing the water through

the filter

If the water is contaminated with fallout, clay can be added to help the

fallout particles to settle out.  The procedure is as follows:

Fill a bucket or other deep container 3/4 full with contaminated water.

Dig pulverized clay or clayey soil from a depth of four or more inches

below ground surface and stir it into the water. Use about 1 inch of dry

clay or clayey soil for every 4" depth of water.  Stir until practically

all of the clay particles are suspended in the water.  Let the clay

settle for at least 6 hours.  This will carry the fallout particles to

the bottom and cover them.  Carefully dip out or siphon the clear water

and disinfect it.

Chemical Treatment

Chlorine:

Chlorine is familiar to most Americans as it is used to treat virtually

all municipal water systems in the United States.  For a long time

chlorine, in the form of Halazone tablets, was used to purify small

batches of water for campers and military troops.  Later questions

emerged about the effectiveness of Halazone, and in 1989, Abbot labs

pulled it off the market.  If Halazone tablets are encountered outside

the US, the nominal shelf life is 6 months, and the dosage is 2 tabs per

liter.  Until recently, there was no chlorine product designed for

wilderness/survival use available in the US.

Chlorine has a number of problems when used for field treatment of

water.  When chlorine reacts with organic material, it attaches itself

to nitrogen containing compounds (ammonium ions and amino acids),

leaving less free chlorine to continue disinfection.  Carcinogenic

trihalomethanes are also produced, though this is only a problem with

long-term exposure.  Trihalomethanes can also be filtered out with a

charcoal filter, though it is more efficient to use the same filter to

remove organics before the water is chlorinated.  Unless free chlorine

is measured, disinfection can not be guaranteed with moderate doses of

chlorine.  One solution is  superchlorination, the addition of far more

chlorine  than is needed.  This must again be filtered through activated

charcoal to remove the large amounts of chlorine, or hydrogen peroxide

can be added to drive the chlorine off.  Either way there is no residual

chlorine left to prevent recontamination.  This isn't a problem if the

water is to be used at once.

Chlorine is sensitive to both the pH and temperature of the treated

water.  Temperature slows the reaction for any chemical treatment, but

chlorine treatment is particularly susceptible to variations in the pH

as at lower pHs, hypochlorous acid is formed, while at higher pHs, it

will tend to dissociate into hydrogen and chlorite ions, which are less

effective as a disinfectant.  As a result, chlorine effectiveness drops

off when the pH is greater than 8

Chlorine, like iodine, will not kill Cryptosporidia.

Methods of chlorine treatment:

Bleach:  Ordinary household bleach (such as Clorox) in the US contains

5.25% sodium hypochlorite (NaOCL) and can be used to purify water if it

contains no other active ingredients, scents, or colorings.  Bleach is

far from an ideal source due to its bulkiness (only 5% active

ingredient), and the instability over time of the chlorine content in

bleach.  Chlorine loss is farther increased by agitation or exposure to

air.  One source claims chlorine loss from a 5% solution at 10% over 6

months if stored at 70deg F.  Nevertheless, this may be the only

chemical means available to purify water, and it is far better than

nothing. Normal dosage is 8 drops (0.4 ml) per gallon. Allow the treated

water to sit for 30 min., and if there isn't a slight chlorine smell,

retreat. Note: USP standard medicine droppers are designed to dispense

0.045-0.055 ml  per drop.  Use of other solvents or some chemicals can

change this.  The dropper can be calibrated against a graduated cylinder

for greater accuracy.

Some small treatment plants in Africa produce their own sodium

hypochlorite on site from the electrolysis of brine.  Power demands

range from 1.7 to 4 kWh per lb. of NaOCL. 2 to 3.5 lbs. of salt are

needed for each pound of NaOCL.  These units are fairly simple and are

made in both the US and the UK.  Another system, designed for China,

where the suitable raw materials were mined or manufactured locally,

used a reaction between salt, manganese dioxide, and sulfuric acid to

produce chlorine gas.  The gas was then allowed to react with slaked

lime to produce a bleaching powder that could then be used to treat

water.  A heat source is required to speed the reaction up.

AquaCure:  Designed for the South African military, these tablets

contain chlorine and alum.  The alum causes the suspended solids to

flocculate and the chlorine adds 8 PPM chlorine.  This is a great way to

treat turbid water, though it will leave a lot of chlorine in clear

water (The one tablet/liter could be halved for clear water.)

The US distributor for Aqua Cure is:

Safesport Manufacturing

Box 11811

Denver, CO  80211

1 800 433 6506

Bleaching Powder (Chlorinated Lime):  Can also be purchased and used as

a purification means if nothing else is available.  Bleaching powder is

33-37% chlorine when produced, but losses its chlorine rapidly,

particularly when exposed to air, light or moisture.

Calcium Hypochlorite:  Also known as High Test Hypochlorite.  Supplied

in crystal form, it is nearly 70% available chlorine.  One product, the

Sanitizer (formally the Sierra Water Purifier) uses these crystals to

superchlorinate the water to insure pathogens were killed off, then

hydrogen peroxide is added to drive off the residual chlorine.  This is

the most effective method of field chlorine treatment.  The US military

and most aid agencies also use HTH to treat their water, though a test

kit, rather than superchlorination, is used to insure enough chlorine is

added.  This is preferable for large-scale systems as the residual

chlorine will prevent recontamination

Usually bulk water treatment plants first dilute to HTH to make a 1%

working solution at the rate of 14g HTH per liter of water.  While

testing to determine exact chlorine needs are preferable, the solution

can be used at the dose rate of 8 drops/gallon, or for larger

quantities, 1 part of 1% solution to 10,000 parts clear water.  Either

of these doses will result in 1 PPM chlorine and may need to be

increased if the water wasn't already filtered by other means.

When test kits are available, the WHO standard is a residual chlorine

level of 0.2 to 0.5 mg/l after a 30 min. contact time.  The may require

as much as 5 mg/l of chlorine to be added to the raw water.

Iodine:

Iodine's use as a water purification method emerged after WW2, when the

US military was looking for a replacement for Halazone tablets. Iodine

was found to be in many ways superior to chlorine for use in treating

small batches of water.  Iodine is less sensitive to the pH and organic

content of water, and is effective in lower doses.  Some individuals are

allergic to iodine, and there is some question about long term use of

iodine.  The safety of long-term exposure to low levels of iodine was

proven when inmates of three Florida prisons were given water

disinfected with 0.5 to 1.0 PPM iodine for 15 years.  No effects on the

health or thyroid function of previously healthy inmates was observed.

Of 101 infants born to prisoners drinking the water for 122- 270 days,

none showed detectable thyroid enlargement.  However 4 individuals with

preexisting cases of hyperthyroidism became more symptomatic while

consuming the water.

Nevertheless experts are reluctant to recommend iodine for long term

use.  Average American iodine intake is estimated at 0.24 to 0.74

mg/day, higher than the RDA of 0.4 mg/day. Due to a recent National

Academy of Science recommendation that iodine consumption be reduced to

the RDA, the EPA discourages the use of iodized salt in areas where

iodine is used to treat drinking water.

Iodine is normally used in doses of 8 PPM to treat clear water for a 10

minute contact time. The effectiveness of this dose has been shown in

numerous studies.  Cloudy water needs twice as much iodine or twice as

much contact time.  In cold water (Below 41deg F or 5deg C) the dose or

time must also be doubled.  In any case doubling the treatment time will

allow the use of half as much iodine

These doses are calculated to remove all pathogens (other than

cryptosporida) from the water.  Of these, giardia cysts are the hardest

to kill, and are what requires the high level of iodine. If the cysts

are filtered out with a microfilter (any model will do since the cysts

are 6 microns), only 0.5 PPM is needed to treat the resulting water .

Water treated with iodine can have any objectionable taste removed by

treating the water with vitamin C (ascorbic acid), but it must be added

after the water has stood for the correct treatment time.  Flavored

beverages containing vitamin C will accomplish the same thing.  Sodium

thiosulfate can also be used to combine with free iodine, and either of

these chemicals will also help remove the taste of chlorine as well.

Usually elemental iodine can't be tasted below 1 PPM, and below 2 PPM

the taste isn't objectionable.  Iodine ions have an even higher taste

threshold of 5 PPM.  Note that removing the iodine taste does not reduce

the dose of iodine ingested by the body

Sources of Iodine:

Tincture of Iodine:  USP tincture of iodine contains 2% iodine and 2.4%

sodium iodide dissolved in 50% ethyl alcohol.  For water purification

use, the sodium iodide has no purification effect, but contributes to

the total iodine dose.  Thus it is not a preferred source of iodine, but

can be used if other sources are not available.  0.4 cc's (or 8 drops)

of USP tincture (2% iodine) added to a liter of water will give the 8

mg/l (same as 8 PPM).  If the iodine tincture isn't compounded to USP

specs,  then you will have to calculate an equal dose based on the

iodine concentration.

Lugol's solution:  Contains 5% iodine and 10% potassium iodide. 0.15 cc

(3 drops) can be added per liter of water, but 3 times more iodine is

consumed compared to sources without iodide.

Betadyne (povidone iodine)  Some have recommended 8 drops of 10%

povidone iodine per liter of water as a water treatment method, claiming

that at low concentrations povidone iodine can be regarded as a solution

of iodine.  One study  indicated that at 1:10,000 dilution (2

drops/liter), there was 2 PPM iodine, while another study resulted in

conflicting results.  However, at 8 drops/liter, there is little doubt

that there is an antimicrobial effect.  The manufacturer hasn't spent

the money on testing this product against EPA standard tests, but in

other countries it has been sold for use in field water treatment.

Kahn-Vassher solution.  By adding a sufficient amount of iodine crystals

to a small bottle, an almost unlimited supply of saturated iodine

solution can be  produced.  As long as crystals remain in the bottle,

the solution is saturated.  Concentration of the iodine is dependent of

temperature, either condition at ambient temperature can be assumed, or

commercial models such as Polar Pure incorporate a liquid crystal

thermometer to determine dose

One criticism of this method is the chance of decanting iodine crystals

into the water being treated.  This isn't that much of a problem as

iodine is very weakly toxic, but the Polar Pure incorporates a collar

into the neck of the bottle to help prevent this.  Another disadvantage

to this method is that the saturated iodine solution must be kept in

glass bottles, and is subject to freezing, but this is hardly an

insurmountable problem.  Freezing, of course, doesn't affect the

crystals.

This is the method I use, but I do use the commercial Polar Pure bottle,

and refill it as necessary with USP crystals.  During a crisis, or

extended camping trips I would microfilter the water first, so a much

lower dose of iodine is needed.

With the Polar Pure bottle, dosage information is provided.  Otherwise a

1 oz bottle can be used to carry the solution.  The bottle is filled

with water after use.  At the next use, 1/2 of the supernate (15 cc) is

poured off into a liter of water.  At 68deg F, this will yield  a dose

of 9 mg/l.  To use this method with a microfilter to get a 0.5 PPM

concentration, either large batches of water need to be treated (1/2 oz

to 4.5 gallons would be 0.5 PPM), or a TB syringe or medicine dropper

can be used to measure doses.  A USP medicine dropper should give 20

drops per ml.

Iodine can also be dissolved in alcohol to make a solution of known

concentration.  I am not aware of any commercial products, but a

pharmacy could compound one for you, or you could do it your self.  One

suggested formula is 8g iodine/100 cc ethyl alcohol which yields enough

solution to disinfect 250 gallons of water.  At the rate of 0.1 cc (2

drops)/liter to give a concentration of 8 mg/l

Tetraglycine hydroperiodide  (e.g. Potable Aqua)  This is the form of

iodine used by the US military for field treatment of water in canteen

sized batches.  Usual dose in one tablet per quart of water to give a

concentration of 8 mg/l.  Two tablets are used in cloudy or cold water

or contact time is doubled.  The major downside of this product is that

the product will loose its iodine rapidly when exposed to the air.

According to the manufacturer, they have a near indefinite life when

sealed in the original bottle, but probably should be discarded within a

few months of opening.  The tablets will change color from gun metal

gray to brown as they lose the iodine, and you should see a brown tint

to the water after treating.

Iodine Resin Filter:  Some commercial microfilters incorporate an iodine

resin stage to kill viruses and bacteria, with out putting as much

iodine in the water as if it had been added to the raw water.  A few

products rely exclusively on an iodine resin stage.  Downside of these

filters are their fragile nature, dependency of effectiveness on flow

rate and the inability to identify when they need to be discarded.  If

you are going to use one where the water is known to be contaminated

with viruses, then one of the better known brands such as the PUR or

Sweetwater Viraguard is recommended.  More than one pass through the

filter may be necessary in cold weather.

Resins do have the advantage of producing less iodine in the water for

the same antimicrobial effect as for the most part, they only release

iodine when contacted by a microbe. The downside is that physical

contact between the microbe and the resin is needed.

Silver:

Silver has been suggested by some for water treatment and may still be

available outside the US.  Its use is currently out of favor due to the

EPA's establishment of a 50 ppb  MCL (Maximum Contaminate Level) limit

on silver in drinking water.  This limit is set to avoid argyrosis, a

cosmetic blue/gray staining of the skin, eyes, and mucous membranes.  As

the disease requires a net accumulation of 1 g of silver in the body,

one expert calculated that you could drink water treated at 50 ppb for

27 years before accumulating 1 g.  Silver has only be proven to be

effective against bacteria and protozoan cysts, though it is quite

likely also effective against viruses.

Silver can be used in the form of a silver salt, commonly silver

nitrate, a colloidal suspension, or a bed of metallic silver.

Electrolysis can also be used to add metallic silver to a solution

Some evidence has suggested that silver deposited on carbon block

filters can kill pathogens without adding as much silver to the water .

Katadyn markets a silver based water treatment product called Micropur.

The manufacturer recommends a 2 hr contact time at a dose of 1 tab per

liter and states the product is “For the disinfection and storage of

clear water.  Reliably kills bacterial agents of enteric diseases, but

not worm eggs, amoeba, or viruses.  Neutral to taste...insure protection

against reinfection for 1-6 months.”;  The following forms are

available:

Micropur Tablets	MT1 1 tablets/qt	25 gal

MT2 1 tablet/5qts   62.5 gal

Micropur Fluid	MF 75 10 drops/gal	  75 gals

MF250 "          "   250 gals

Micropur Crystal	MC250	1 packet/gal		    250 gal

MC 2500     1 spoon/25 gal         2500 gal

MC12500     1 spoon/250 gal       12500 gal

Potassium Permanganate:

Potassium Permanganate is no longer commonly used in the developed world

to kill pathogens.  It is much weaker than the alternatives, more

expensive, and leaves a objectionable pink or brown color.  If it must

be used, 1 gram per liter would probably be sufficient against bacteria

and viruses (no data is available on it effectiveness against protozoan

cysts.

Hydrogen Peroxide:

Hydrogen Peroxide can be used to purify water if nothing else is

available.  Studies have shown of 99 percent inactivation of poliovirus

in 6 hr with 0.3 percent hydrogen peroxide and a 99% inactivation of

rhinovirus with a 1.5% solution in 24 minutes.  Hydrogen Peroxide is

more effective against bacteria, though Fe+2 or Cu+2 needs to be present

as a catalyst to get a reasonable concentration-time product.

Coagulation/Flocculation agents:

While flocculation doesn't kill pathogens, it will reduce their levels

along with removing particles that could shield the pathogens from

chemical or thermal destruction, and organic matter that could tie up

chlorine added for purification.  60-98% of coliform bacteria, 65-99% of

viruses, and 60-90% of giardia will be removed from the water, along

with organic matter and heavy metals.

Some of the advantages of coagulation/flocculation can be obtained by

allowing the particles to settle out of the water with time

(sedimentation), but it will take a while for them to do so.  Adding

coagulation chemicals such as alum will increase the rate at which the

suspended particles settle out by combining many smaller particles into

larger floc which will settle out faster. The usual dose for alum is

10-30 mg/liter of water.  This dose must be rapidly mixed with the

water, then the water must be agitated for 5 minutes to encourage the

particles to form flocs.  After this at least 30 minutes of settling

time is need for the flocs to fall to the bottom, and them the clear

water above the flocs may be poured off.  Most of the flocculation agent

is removed with the floc, nevertheless some question the safety  of

using alum due to the toxicity of the aluminum in it. There is little to

no scientific evidence to back this up.  Virtually all municipal plants

in the US dose the water with alum.

In bulk water treatment, the alum dose can be varied until the idea dose

is found.  The needed dose varies with the pH of the water and the size

of the particles.  Increase turbidity makes the flocs easier to produce

not harder, due to the increased number of collisions between particles.

Treatments requiring electricity:

Ozone:

Ozone is used extensively in Europe to purify water.  Ozone, a molecule

composed of 3 atoms of oxygen rather than two, is formed by exposing air

or oxygen to a high voltage electric arc. Ozone is much more effective

as a disinfectant than chlorine, but no residual levels of disinfectant

exist after ozone turns back into O2. (one source quotes a half life of

only 120 minutes in distilled water at 20deg C).  Ozone is expected to

see increased use in the US as a way to avoid the production of

trihalomethanes.  While ozone does break down organic molecules,

sometimes this can be a disadvantage as ozone treatment can produce

higher levels of smaller molecules that provide an energy source for

microorganisms.  If no residual disinfectant is present (as would happen

if ozone were used as the only treatment method), these microorganisms

will cause the water quality to deteriorate in storage.

Ozone also changes the surface charges of dissolved organics and

colloidially suspended particles.  This causes microflocculation of the

dissolved organics and coagulation of the colloidal particles

UV light

Ultraviolet light has been known to kill pathogens for a long time.  A

low pressure mercury bulb emits between 30 to 90 % of its energy at a

wave length of 253.7 nm, right in the middle of the UV band.  If water

is exposed to enough light, pathogens will be killed.  The problem is

that some pathogens are hundreds of times less sensitive to UV light

than others.  The least sensitive pathogens to UV are protozoan cysts.

Several studies show that Giardia will not be destroyed by many

commercial UV treatment units.  Fortunately these are the easiest

pathogens to filter out with a mechanical filter

The efficacy of UV treatment is very dependent on the turbidity of the

water.  The more opaque the water is, the less light that will be

transmitted through it.  The treatment units must be run at the designed

flow rate to insure sufficient exposure, as well as insure turbulent

flow rather than plug flow.

Another problem with UV treatment is that the damage done to the

pathogens with UV light can be reversed if the water is exposed to

visible light (specifically 330-500 nm) through a process known as

photoreactivation.

UV treatment, like ozone or mechanical filtering leaves no residual

component in the water to insure its continued disinfection.  Any

purchased UV filter should be checked to insure it at least complies

with  the 1966 HEW standard of 16 mW.s/cm^2 with a maximum water depth

of 7.5 cm. ANSI/NSF require 38 mWs/cm^2 for primary water treatment

systems.  This level was chosen to give better than 3 log (99.9%)

inactivation of Bacillus subtillis.  This level is of little use against

Giardia, and of no use against Crypto.

The US EPA explored UV light for small scale water treatment plants and

found it compared unfavorably with chlorine due to 1) higher costs, 2)

lower reliability, and 3) lack of a residual disinfectant.

Questionable or Dangerous methods of water treatment.

1) Aerobic 07:  Also sold as Aerobic Oxygen.  The company refuses to

release the disinfectant.  It maybe chlorine dioxide, a well known, if

somewhat unstable, disinfectant.  The company has shown company

sponsored tests showing effectiveness against viruses and bacteria (but

not against Giardia).  No independent testing has been performed, nor

has anybody provided concentration-time data for the product.

2) Survival Straw:  This product claims to destroy and eliminate

impurities including bacteria, protozoa. fungi, chemicals and heavy

metals using a matrix of metal alloy.  The manufacturer claims the

product’s media meets EPA and FDA specs, which is no indication of the

filter's effectiveness.  The filter violates a number of laws of physics

since it claims that it destroys heavy metals and pathogens without

filtering them.